- Make at least 0.5mm clearance both sides in two circled regions

- Holes are undersized.  1/4in bolt is 6.35mm OD, and the holes are sized exactly the same.  Will have difficulty pushing through.  Recommend minimum 6.6mm OD for holes.

Same for here as well:

- These holes are sized exactly same as screw.  Recommend 5.11mm OD for three holes here

- Concerned regarding clearance and assembly.  The bolt head is 0.505in max OD, meaning it will be difficult to assemble with a screw.  Recommend moving bolt position down by another 0.5cm.

- Increase thickness to 10mm.  The screws will hit the end of the motor below this.

- Increase diameter to at least 2.6in.  Currently is undersized at 32mm radius

- Based on assumption that small holes are using 10-32 bolt (metric), the gap is too small for head of bolt.  In addition, the holes should be clearance holes.  Currently all small diameters are 4.8mm.  Should be minimum 10.8mm.